Repository & Pipeline Structure

Full folder and file layout — common layer, 15 market-gate phases, configuration, tests, and runtime artifacts.

Alqueva-PSP-PV-BESS-24hr-Energy-Trading-DA-IDA-aFRR-mFRR-Optimizer/ │ ├── run_production.py # ◄ Master orchestrator — all 15 phases │ ├── common_layer/ # Shared foundation — imported by every phase │ ├── configuration/ │ │ ├── config_loader.py # load_config() → AppConfig │ │ ├── market_config.py # MarketConfig dataclass │ │ ├── plant_config.py # PlantConfig dataclass │ │ └── solver_config.py # SolverConfig dataclass │ │ │ ├── optimisation_model/ │ │ ├── core_milp_builder.py # CoreModelMeta, build_milp() — ONE model for all gates │ │ ├── core_milp_solver.py # solve_milp(), SolveError — CPLEX → HiGHS → CBC │ │ ├── ida_reoptimiser.py # optimise_ida() — freeze hours, re-solve │ │ ├── reserve_offer_builder.py # build_afrr_offers(), build_mfrr_offers() │ │ ├── activation_ramp_tracker.py # ramp-corrected eff_isp_h for settlement │ │ └── reserve_activation.py # reserve activation helpers │ │ │ ├── physical_plant_models/ │ │ ├── psp_turbine_pump_model.py # PSPModel, UnitDispatch (4 Francis units) │ │ ├── pv_production_model.py # PVModel (5 MWp, temperature derate, degradation) │ │ ├── bess_model.py # BESSModel, BESSDispatch (1 MW / 2 MWh) │ │ ├── reservoir_model.py # ReservoirModel (Alqueva + Pedrógão) │ │ ├── fcr_headroom_model.py # FCRHeadroomModel (reserved — never sold) │ │ └── reservoir_activation_checker.py # validates long-pumping sequences │ │ │ ├── database/ │ │ ├── position_store.py # PositionStore → runtime/db/positions.db │ │ ├── reserve_store.py # ReserveStore → runtime/db/reserve.db │ │ ├── realtime_store.py # DeliveryStore, ActivationStore → realtime.db │ │ ├── component_store.py # ComponentStore → runtime/components/<date>.json │ │ ├── audit_store.py # AuditStore (read-only) → audit_<date>.jsonl │ │ └── schema_validator.py # input schema validation │ │ │ ├── gate_scheduler/ │ │ ├── gate_scheduler.py # GateScheduler — CET gate-time resolver │ │ └── gate_trigger_spec.py # trigger definitions from market.yaml │ │ │ └── utilities/ │ ├── audit_logger.py # AuditLogger — append-only JSONL trail │ ├── timezone_utils.py # CET (OMIE) ↔ WET/CET (plant) conversions │ ├── date_utils.py # delivery date parsing, D-1 calculations │ └── logging_utils.py # phase-prefixed logger setup │ ├── phase_1_da_day_ahead_bidding/ # ── Phase 1, Day-Ahead ───────────────────── │ ├── run_da.py # entry point │ ├── da_price_pv_inflow_forecasting/ │ │ ├── da_price_forecaster.py # ML DA price forecast │ │ ├── da_price_train_val_test.py │ │ ├── pv_power_forecaster.py # PV production forecast │ │ ├── pv_train_val_test.py │ │ ├── reservoir_inflow_forecaster.py # natural inflow forecast │ │ ├── reservoir_inflow_train_val_test.py │ │ ├── omie_da_price_loader.py # live OMIE DA price loader │ │ └── ml_train_val_test_common.py # shared ML utilities │ ├── da_bid_formatting/ │ │ ├── da_bid_formatter.py # formats OMIE bid payload │ │ └── da_bid_checker.py # physical bid validation │ ├── risk_and_bid_validation/ │ │ └── pre_trade_risk_checker.py # pre-trade risk limits │ └── trader_approval/ │ └── trader_approval_prompt.py # [A]/[R] interactive prompt │ ├── phase_2a_ida1_intraday_auction_1/ # ── Phase 2A, IDA1 ────────────────────── │ ├── run_ida1.py # entry point │ ├── ida1_price_forecasting/ │ │ ├── ida1_price_forecaster.py │ │ ├── ida1_price_train_val_test.py │ │ └── omie_ida1_price_loader.py │ ├── ida1_milp_reoptimiser/ │ │ └── ida1_reoptimiser.py # freeze DA, re-solve H1–H24 │ └── ida1_bid_formatting/ │ └── ida1_bid_formatter.py # SIDC delta bid payload │ ├── phase_2b_ida2_intraday_auction_2/ # ── Phase 2B, IDA2 ────────────────────── │ ├── run_ida2.py │ ├── ida2_price_forecasting/ │ │ ├── ida2_price_forecaster.py │ │ ├── ida2_price_train_val_test.py │ │ └── omie_ida2_price_loader.py │ ├── ida2_milp_reoptimiser/ │ │ └── ida2_reoptimiser.py # freeze H1–H2, re-solve H3–H24 │ └── ida2_bid_formatting/ │ └── ida2_bid_formatter.py │ ├── phase_2c_ida3_intraday_auction_3/ # ── Phase 2C, IDA3 ────────────────────── │ ├── run_ida3.py │ ├── ida3_price_forecasting/ │ │ ├── ida3_price_forecaster.py │ │ ├── ida3_price_train_val_test.py │ │ └── omie_ida3_price_loader.py │ ├── ida3_milp_reoptimiser/ │ │ └── ida3_reoptimiser.py # freeze H1–H11, re-solve H12–H24 │ └── ida3_bid_formatting/ │ └── ida3_bid_formatter.py │ ├── phase_2d_xbid_continuous_intraday/ # ── Phase 2D, XBID ────────────────────── │ ├── run_xbid.py │ ├── xbid_price_forecasting/ │ │ ├── xbid_price_forecaster.py │ │ ├── xbid_price_train_val_test.py │ │ ├── xbid_price_loader.py │ │ └── create_xbid_training_data.py │ ├── xbid_milp_optimiser/ │ │ └── xbid_optimiser.py # per-order caps, H-1 rolling │ └── xbid_bid_formatting/ │ └── xbid_bid_formatter.py │ ├── phase_3a_afrr_automatic_frequency_reserve/ # ── Phase 3A, aFRR ────────────────────── │ ├── run_afrr.py │ ├── afrr_price_forecasting/ │ │ ├── afrr_price_forecaster.py │ │ ├── afrr_price_train_val_test.py │ │ ├── picasso_afrr_price_loader.py # PICASSO live price loader │ │ └── create_afrr_training_data.py │ └── afrr_reserve_offer_builder/ │ ├── afrr_offer_builder.py # headroom → symmetric up/dn offers │ └── afrr_offer_checker.py # FAT deliverability, cap ≤ 250 EUR/MW │ ├── phase_3b_mfrr_manual_frequency_reserve/ # ── Phase 3B, mFRR ────────────────────── │ ├── run_mfrr.py │ ├── mfrr_price_forecasting/ │ │ ├── mfrr_price_forecaster.py │ │ ├── mfrr_price_train_val_test.py │ │ ├── mari_mfrr_price_loader.py # MARI live price loader │ │ └── create_mfrr_training_data.py │ └── mfrr_reserve_offer_builder/ │ ├── mfrr_offer_builder.py │ └── mfrr_offer_checker.py │ ├── phase_4a_isp_real_time_dispatch/ # ── Phase 4A, Real-Time ───────────────────── │ ├── run_realtime.py │ ├── isp_setpoint_dispatch/ │ │ ├── psp_setpoint_dispatcher.py # PSP unit setpoints per ISP │ │ └── bess_setpoint_dispatcher.py # BESS setpoints per ISP │ ├── isp_activation_tracking/ │ │ └── isp_position_tracker.py # scheduled vs actual per ISP │ └── telemetry/ │ └── ren_isp_signal_loader.py # REN telemetry feed │ ├── phase_4b_afrr_activation_response/ # ── Phase 4B, aFRR Activation ──────────────── │ ├── run_afrr_activation.py │ ├── afrr_setpoint_dispatch/ │ │ └── afrr_activation_handler.py # ramp, min hold 2 ISPs, eff_isp_h │ └── afrr_activation_tracking/ │ └── afrr_activation_logger.py │ ├── phase_4c_mfrr_activation_response/ # ── Phase 4C, mFRR Activation ──────────────── │ ├── run_mfrr_activation.py │ ├── mfrr_setpoint_dispatch/ │ │ └── mfrr_activation_handler.py # ramp, min hold 3 ISPs, eff_isp_h │ └── mfrr_activation_tracking/ │ └── mfrr_activation_logger.py │ ├── phase_5a_da_ida_settlement/ # ── Phase 5A, Energy Settlement ─────────────── │ ├── run_energy_settlement.py │ └── energy_settlement_calculation/ │ ├── da_settlement_calculator.py # DA volume × OMIE settlement price │ ├── ida_settlement_calculator.py # IDA delta per gate, no double-counting │ └── omie_settlement_price_loader.py # final OMIE settlement prices │ ├── phase_5b_reserve_settlement/ # ── Phase 5B, Reserve Settlement ────────────── │ ├── run_reserve_settlement.py │ └── reserve_settlement_calculation/ │ ├── afrr_settlement_calculator.py # capacity (hourly) + activation, eff_isp_h │ ├── mfrr_settlement_calculator.py # reuses generic settle_reserve() │ └── ren_reserve_settlement_loader.py # PICASSO + MARI invoice loader │ ├── phase_5c_imbalance_settlement/ # ── Phase 5C, Imbalance Settlement ──────────── │ ├── run_imbalance_settlement.py │ ├── imbalance_price_and_volume/ │ │ ├── imbalance_volume_calculator.py # actual − scheduled per ISP │ │ └── ren_imbalance_price_loader.py # REN post-delivery prices │ └── imbalance_settlement_calculation/ │ └── imbalance_settlement_calculator.py # long→DA×0.85, short→DA×1.20 │ ├── phase_5d_analytics_and_reporting/ # ── Phase 5D, Analytics ───────────────────── │ ├── run_analytics.py │ ├── analytics_and_kpis/ │ │ ├── daily_pnl_calculator.py # compute_daily_pnl() │ │ ├── revenue_breakdown_analyzer.py # revenue_shares() by market stream │ │ ├── kpi_reporter.py # compute_kpis() — 10 KPI sections │ │ └── operational_analytics.py │ └── daily_excel_reports/ │ ├── daily_report_exporter.py # export_daily_report() → .xlsx │ ├── dispatch_sheet_builder.py # Dispatch_Hourly (94 cols) │ └── summary_kpi_builder.py # Summary_KPIs sheet │ ├── phase_6_backtesting_and_validation/ # ── Phase 6, Backtesting ───────────────────── │ ├── run_backtest.py │ ├── backtest_engine/ │ │ ├── backtest_runner.py # historical date-range replay │ │ └── historical_data_loader.py # load historical prices / inflows │ ├── forecast_and_model_validation/ │ │ ├── price_forecast_validator.py # DA / IDA price forecast accuracy │ │ ├── pv_forecast_validator.py # PV production forecast accuracy │ │ └── milp_solution_quality_checker.py # MIP gap, feasibility, solve time │ ├── risk_analytics/ │ │ └── portfolio_risk_metrics.py # VaR, CVaR, revenue volatility │ └── backtest_excel_reports/ │ └── backtest_report_exporter.py │ ├── figures/ # ── 9 Production Figures ─────────────────────────────── │ └── __init__.py # generate(date) → all 9 figures at 600 DPI │ ├── config/ # ── Configuration ───────────────────────────────────── │ ├── market.yaml # gate times, IDA dates, ISP, FAT, bid limits │ ├── plant.yaml # PSP / PV / BESS / reservoir, head model │ ├── solver.yaml # CPLEX → HiGHS → CBC, MIP gap, time limits │ └── run.yaml # date, mode, data source, phase flags │ ├── tests/ # ── Test Suite (pytest) ────────────────────────────── │ ├── conftest.py # shared fixtures │ ├── test_e2e_chain.py # end-to-end pipeline chain │ ├── test_milp_physics.py # MILP constraint physics │ ├── test_ida_frozen.py # IDA hour-freezing logic │ ├── test_settlement.py # settlement calculations │ ├── test_reserve_checker.py # reserve offer validation │ ├── test_reserve_market_deep.py # deep reserve market tests │ ├── test_reserve_hidden_invariants.py # PR-11 no-double-selling invariants │ ├── test_reserve_realtime_delivery.py # activation + delivery chain │ ├── test_checker_negative.py # negative / edge-case physical checks │ └── test_bug_regressions.py # regression suite │ ├── runtime/ # ── Auto-created at first run ──────────────────────── │ ├── db/ │ │ ├── positions.db # SQLite — energy positions (DA, IDA, XBID) │ │ ├── reserve.db # SQLite — aFRR / mFRR capacity offers │ │ └── realtime.db # SQLite — per-ISP delivery & activations │ ├── audit/ │ │ └── audit_YYYY-MM-DD.jsonl # append-only audit trail (one JSON per event) │ ├── components/ │ │ └── components_YYYY-MM-DD.json # per-component DA dispatch results │ └── reports/ │ └── daily_report_YYYY-MM-DD.xlsx # 5-sheet Excel report │ ├── docs/ # ── Architecture Diagrams ───────────────────────────── │ ├── pipeline_architecture.png # pipeline diagram (2× retina, 940 px) │ └── pipeline_architecture.svg # same diagram as scalable SVG │ └── requirements.txt # pip install -r requirements.txt